###### ANEXA 11

# FOAIE DATE ROBINET CU SUPAPĂ DE SIGURANŢĂ DN 80 / DN 100

| GENERALITIES | 1 | TAG NUMBER: |  |  |  |  |  |  |  |  |  | PRV 003 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 | P&ID NUMBER: |  |  |  |  |  |  |  |  |  | BO-0227 |  |  |  |
|  | 3 | LINE/EQUIPMENT NUMBER: |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  | 4 | SERVICE: |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  | 5 | MANUFACTURER: |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  | 6 | MODEL/PART NO.: |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  | 7 | DESIGN TYPE: |  |  |  |  |  |  |  |  | (Note 1) | RELIEF VALVE |  |  |  |
| PROCESS CONDITIONS | 8 | FLUID: |  |  |  |  |  |  |  |  |  | LIQUID |  |  |  |
|  | 9 | CORROSIVE COMPOUNDS: |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  | 10 | RELIEVING RATE: |  |  |  |  |  |  |  |  | Sm3/h | - |  |  |  |
|  | 11 | SET PRESSURE: |  |  |  |  |  |  |  |  | barg | 45 |  |  |  |
|  | 12 | OVERPRESSURE - % |  |  |  |  |  |  |  |  |  | 10 |  |  |  |
|  | 13 | MAX. RELIEVING PRESSURE: |  |  |  |  |  |  |  |  | barg | 50.51 |  |  |  |
|  | 14 | RELIEVING TEMPERATURE (T): |  |  |  |  |  |  |  |  | °C | 30 |  |  |  |
|  | 15 | DOWNSTREAM PRESSURE (CONSTANT): |  |  |  |  |  |  |  |  | barg | - |  |  |  |
|  | 16 | DOWNSTREAM PRESSURE (VARIABLE): |  |  |  |  |  |  |  |  | barg | - |  |  |  |
|  | 17 | LIQUID FLOW |  |  |  |  |  |  |  |  | m3/h | 250 |  |  |  |
|  | 18 |  | - SPECIFIC GRAVITY @ T: |  |  |  |  |  |  |  |  | - |  |  |  |
|  | 19 |  | - VISCOSITY @ T: |  |  |  |  |  |  |  | cP | 0,1395 @ 20°C |  |  |  |
|  | 20 | VAPOUR FLOW |  |  |  |  |  |  |  |  | Nm3/h | - |  |  |  |
|  | 21 |  | - MOL WEIGHT: |  |  |  |  |  |  |  |  | - |  |  |  |
|  | 22 |  | - COMPRESSIBILITY - Z: |  |  |  |  |  |  |  |  | - |  |  |  |
|  | 23 |  | - VISCOSITY @ T: |  |  |  |  |  |  |  | cP | - |  |  |  |
|  | 24 |  | - SPECIFIC HEAT RATIO-Cp/Cv: |  |  |  |  |  |  |  |  | - |  |  |  |
|  | 25 |  |  |  |  |  |  |  |  |  |  | MIN. | NORMAL |  | MAX. |
|  | 26 | OPERATING PRESSURE: |  |  |  |  |  |  |  |  | barg |  | 30 |  |  |
|  | 27 | OPERATING TEMPERATURE: |  |  |  |  |  |  |  |  | °C | 10 | 15 |  | 30 |
|  | 28 | VALVE PED CATEGORY: |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  | 29 | FLUID PED CLASSIFICATION: |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  | 30 | AMBIENT TEMPERATURE: |  |  |  |  |  |  |  |  | °C | - 30 / + 39,4 |  |  |  |
| VALVE | 31 | BODY TYPE: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 32 | INLET: |  |  | TYPE |  | STYLE |  |  |  |  | FLANGED |  | Clasa 300 |  |
|  | 33 |  |  |  | SIZE |  | RATING |  |  |  |  | 3" |  |  |  |
|  | 34 | OUTLET: |  |  | TYPE |  | STYLE |  |  |  |  | FLANGED |  | Clasa 150 |  |
|  | 35 |  |  |  | SIZE |  | RATING |  |  |  |  | 4” |  |  |  |
|  | 36 | BONNET TYPE: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 37 | SEAT TYPE: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 38 | CALCULATED DISCHARGE AREA: |  |  |  |  |  |  |  |  | mm2 | 1035.58 |  |  |  |
|  | 39 | ORIFICE DESIGNATOR |  |  |  |  | SELECTED DISCHARGE AREA, mm2 |  |  |  |  | K |  | 1186 |  |
|  | 40 | CONVENTIONAL, BELLOWS, PILOT OPERATED: |  |  |  |  |  |  |  |  |  | - |  |  |  |
| MATERIALS | 41 | BODY : |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 42 | BONNET: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 | SEAT&DISC: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 | GUIDE: |  |  |  |  | RINGS: |  |  |  |  |  |  |  |  |
|  | 45 | SPRING: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 46 | BELLOWS: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 47 | SEALS / O-RINGS: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OTHERS | 48 | LIFTING LEVER: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 49 | BACKFLOW PREVENTER: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50 | SILENCER: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 53 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 54 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |